Communication Protocol of

Tamperproof watch

Content

Explanation	2

<1>Commands sent from end user (device server)	3

1.	Login package(uplink number:AP00,responds:BP00)	3

2.	Locating package, GPS+LBS+Status+Base +WIFI combining package (uplink number:AP01，responds:BP01)	3

3.	Multiple bases locating package (uplink number:AP02，responds:BP02)	4

4.	Alarm and Return address Package(uplink number:AP10，responds:BP10)	5

5.	Heartbeat package(uplink number:AP03，responds:BP03)	7

6.	Low battery alarm uploading package（uplink number：AP04，responds：BP04）	8

7.	Timing through bases（uplink number：AP53，responds：BP53）	8

<2>Commands sent from server（server device）	9

1.	GPRS locating data upload intervals（downlink：BP15，respond：AP15）	9

2.	Real-time locating command（long link valid）（downlink：BP16，respond：AP16）	9

3.	Factory reset（downlink：BP17，respond：AP17）	10

4.	Restart the terminal（downlink：BP18，respond：AP18）	10

5.	Power off command（downlink：BP31，respond：AP31）	11

6.	Working Mode（downlink：BP33，respond：AP33）	11

7.	GPS Working time（downlink：BP34，respond：AP34）	12

Explanation

All data packages in this protocol refer to formats below:

Head: IW

Protocol number: Uplink(deviceserver)AP[two digits],Downlink(serverdevice)BP[two digits],same digits show data packages respond.

Parameter: content of data package

Terminator:#

All information related with Chinese use UNICODE, such as address.

All punctuation marks in the data packages (except the punctuation in issued address) are English half Angle.

<1>Commands sent from end user (device server)

- Login package(uplink number:AP00,responds:BP00)

| Example: |  |
| --- | --- |
| IWAP00353456789012345# |  |
| Notice: |  |
| IW: Identifier AP00:Command word 353456789012345: IMEI number of devices, default as 15 digits #:Terminator |  |
| platform responds: | platform replies: IWBP00,20150101125223,8# 20150101125223 is server time service, the format is year month date hour minute second, it’s UTC0 time. 8 is the present time zone of server |
| Notice: | New login package should be sent every time the device connects with server. |

- Locating package, GPS+LBS+Status+Base +WIFI combining package (uplink number:AP01，responds:BP01)

| Example: |  |
| --- | --- |
| IWAP01080524A2232.9806N11404.9355E000.1061830323.8706000908000102,460,0,9520,3671,Home¦74-DE-2B-44-88-8C¦97& Home1¦74-DE-2B-44-88-8C¦97&Home2¦74-DE-2B-44-88-8C¦97& Home3¦74-DE-2B-44-88-8C¦97# |  |
| Notice: |  |
| IW Identifier AP01: Command word 080524: 24th May 2008 A:“A”shows valid data,”V”shows invalid data and will get LBS data 2232.9806N11404.9355E000.1: 22 degrees north latitude 32.9806 points，114 degrees east longitude 04.9355 points，speed is 000.1 km/h，it’s default as 0 if the latitude and longitude are invalid, such as 0000.0000N00000.0000E 061830: GMT 06:18:30 323.87:direction angle is 323.87° 06000908000102: 060 is GSM signal,009 is the number of satellites,080 is battery level, 0 is remaining space,01 is fortification state,02 is working mode, (it shows none setting if fortification and working mode are 00) 460,0,9520,3671 : LBS is base data MCC is country code,460 means China,0:MNC,0 is moving,9520:LAC,decimal,3671,CID, decimal If state in GPS package is V or latitude and longitude are “ 0000.0000N00000.0000E”,it will get LBS data. Home¦74-DE-2B-44-88-8C¦97 : one set of WIFI information，Home is SSID， 74-DE-2B-44-88-8C is MAC address，97 is signal strength，variables are separated by“¦”,wifi information can be multiple sets and are separated by “&”. |  |
| platform responds: | platform replies: IWBP01# |
| Notice: | This message applies to all end users. |

- Multiple bases locating package (uplink number:AP02，responds:BP02)

| Example： |  |
| --- | --- |
| IWAP02,zh_cn,0,7,460,0,9520¦3671¦13,9520¦3672¦12,9520¦3673¦11,9520¦3674¦10,9520¦3675¦9,9520¦3676¦8,9520¦3677¦7,4,1¦D8-24-BD-79-FA-1F¦59&2¦3C-46-D8-6D-CE-01¦81&3¦0C-4C-39-1A-7C-65¦69&4¦70-A8-E3-5D-D7-C0¦65# |  |
| Notice: |  |
| IW Identifier AP02: Command word zh_cn: language notice 0:reply notice，the server doesn’t reply address when it’s 0,and replies address when it’s 1. 7: 7 sets of bases 460: MCC is country code 0: MNC operator code 9520¦3671¦10: LAC¦CID¦dbm shows a set of base information and the numbers should correspond to the number of bases. 10：dbm is signal strength, multiple bases signal strength is suggested to be arranged from high to low then downloaded, signal strength are using absolute values: 150 – abs(dbm) 4: sets of wifi 1¦D8-24-BD-79-FA-1F¦59: ssid¦mac address¦ signal strength，signal strength are using absolute values:150 – abs(signal strength) |  |
| platform responds: | platform replies: IWBP02# If the bit zone is 1 then responds: IWBP02F16D3357025E5753715C3A535753776D275953903100300037003900F753# The reply address information from platform is UNICODE HEX String which contains address（No.1079 Nanhai Road Nanshan District,Shenzhen） |
| Notice: | This message applies to all end users. |

- Alarm and Return address Package(uplink number:AP10，responds:BP10)

| Example： |  |
| --- | --- |
| IWAP10080524A2232.9806N11404.9355E000.1061830323.8706000908000502,460,0,9520,3671,00,zh-cn,00,HOME¦74-DE-2B-44-88-8C¦97&HOME1¦74-DE-2B-44-88-8C¦97&HOME2¦74-DE-2B-44-88-8C¦97&HOME3¦74-DE-2B-44-88-8C¦97# |  |
| Notice: |  |
| IW : Identifier AP10: Command word 080524: 24th May 2008 A: “A”shows valid data,”V”shows invalid data and will get LBS data 2232.9806N11404.9355E000.1: 22 degrees north latitude 32.9806 points，114 degrees east longitude 04.9355 points，speed is 000.1 km/h，it’s default as 0 if the latitude and longitude are invalid, such as 0000.0000N00000.0000E 061830: GMT 06:18:30 323.87: direction angle is 323.87° 06000908000102: 060 is GSM signal,009 is the number of satellites,080 is battery level, 0 is remaining space,01 is fortification state,02 is working mode, (it shows none setting if fortification and working mode are 00) 460,0,9520,3671 : LBS is base data MCC is country code,460 means China,0:MNC,0 is moving,9520:LAC,decimal,3671,CID, decimal 00 is alarm state,00 is no alarm (01：SOS,02：low battery,03：pull-out alarm,04:wearing notice) zh-cn: device language 00:The first one 0:whether the reply address is needed,0:no reply,1 reply. The second 0:whether mobile hyperlink is contained in address information,0 not contained,1 contained Home¦74-DE-2B-44-88-8C¦97 : one set of WIFI information，Home is SSID， 74-DE-2B-44-88-8C is MAC address，97 is signal strength，variables are separated by“¦”,wifi information can be multiple sets and are separated by “&”. |  |
| platform responds: | platform responds BP10,if return address is not needed, there should be space in return package. IWBP106df157335e0253575c71533a53576d7759279053003100300037003953f7002000200068007400740070003a002f002f007700770077002e006700700073002e0063006f006d002f006d00610070002e0061007300700078003f006c00610074003d00320033002e0031003200330026006c006e0067003d003100310033002e003100320033# Content of return address on platform should be UNICODE of HEX and implied, above example shows: No1079 Nanhai Road,Nanshan District,Shenzhen city http://www.gps.com/map.aspx?lat=23.123&lng=113.123 The language and content automatically judge according to langiage inAP10 package, whether to reply hyperlink according to states of AP10. |
| Notice: | This message applies to all end users. |

- Heartbeat package(uplink number:AP03，responds:BP03)

| Example： |  |
| --- | --- |
| IWAP03,06000908000102,5555,30# |  |
| Notice: |  |
| IW: Identifier AP03: Command word Device can use command to keep connection and handle static drift with platform when it’s motionless. 06000908000102: 060 is GSM signal 009 is the number of satellites 080 is battery level 0 is remaining space 01 is fortification state, Here shows by two hexadecimal,00 if no value, the length should be two digits. There’re 8 digits in total and each digit is defined as below (unlisted one is unused states,1 shows effective and 0 shows invalid) Bit0 fortification state<the device is invalid> Bit1 :1 shows the night-light open,0 shows night-light off. 0202 is working mode, (it shows none setting if fortification and working mode are 00) 5555: counting steps. Recount from 0 after reaching 9999 steps (9999 is for an example, here is 2 bytes int and overflow to zero) 30: Rolls frequency |  |
| platform responds: | platform replies: IWBP03# |
| Notice: | This message applies to all end users. |

- Low battery alarm uploading package（uplink number：AP04，responds：BP04）

| Example： |  |
| --- | --- |
| IWAP04075# |  |
| Notice: |  |
| IW: Identifier AP04：Command word 075：After the battery level is less than 75%, battery level uploading packages will be sent when less than75%,50%，25% and 10% to platform. |  |
| platform responds: | platform replies: IWBP04# |
| Notice: | This message applies to all end users. |

- Timing through server（uplink number：AP53，responds：BP53）

| Example： |  |
| --- | --- |
| IWAP53,460,0,9750,3613# |  |
| Notice: |  |
| IW：Identifier AP53 : Command word 460:MCC 0: MNC 9750: LAC 3613: CID |  |
| platform responds: | IWBP53,20160225090909# Return time, format is: yyyyMMddHHmmss |
| Notice: | This message applies to all end users. |

- Temperature（uplink number：APTP，responds：BPTP）

| Example： |  |
| --- | --- |
| IWAPTP,36.0,33,0# |  |
| Notice: |  |
| IW：Identifier APTP: Command word 36.0: current temperature 33: reference base temperature test point |  |
| platform responds: | IWBPTP# |
| Notice: | This message applies to all end users. |

<2>Commands sent from server（server device）

- GPRS locating data upload intervals（downlink：BP15，respond：AP15）

| Example： |  |
| --- | --- |
| IWBP15,353456789012345,080835,300# |  |
| Notice: |  |
| IW：Identifier BP15 : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : command serial number 300 :locating data uploading intervals of the device, unit: second |  |
| device responds： | IWAP15,080835,300# 080835: command serial number 300: the device responds the present setting to sync platform. |
| Notice: | This message applies to all end users. |

- Real-time locating command（long link valid）（downlink：BP16，respond：AP16）

| Example： |  |
| --- | --- |
| IWBP16,353456789012345,080835# |  |
| Notice: |  |
| IW：Identifier BP16 : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : command serial number |  |
| device responds： | IWAP16,080835# 080835: command serial number Uplink AP01 locating data after responds. |
| Notice: | This message applies to all end users. |

- Factory reset（downlink：BP17，respond：AP17）

| Example： |  |
| --- | --- |
| IWBP17,353456789012345,080835# |  |
| Notice: |  |
| IW：Identifier BP17 : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : command serial number |  |
| device responds： | IWAP17,080835# 080835: command serial number |
| Notice: | This message applies to all end users. |

- Restart the terminal（downlink：BP18，respond：AP18）

| Example： |  |
| --- | --- |
| IWBP18,353456789012345,080835# |  |
| Notice: |  |
| IW：Identifier BP18 : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : command serial number |  |
| device responds： | IWAP18,080835# 080835: command serial number The device restart after responds to the command. |
| Notice: | This message applies to all end users. |

- Power off command（downlink：BP31，respond：AP31）

| Example： |  |
| --- | --- |
| IWBP31,353456789012345,080835# |  |
| Notice: |  |
| IW：Identifier BP31 : Command word 353456789012345: IMEI number and the unique ID of the device 080835: command serial number |  |
| device responds： | IWAP31,080835# 080835: command serial number |
| Notice: | This message applies to all end users. |

- Working Mode（downlink：BP33，respond：AP33）

| Example： |  |
| --- | --- |
| IWBP33,353456789012345,080835,1# |  |
| Notice: |  |
| IW：Identifier BP33 : Command word 353456789012345: IMEI number and the unique ID of the device 080835: command serial number 1：working mode of the device，1：normal mode，2：Power-saving mode，3：emergency mode |  |
| device responds： | IWAP33,080835,1# 080835: command serial number 1：setting information to sync server |
| Notice: | This message applies to all end users. |

- GPS Working time（downlink：BP34，respond：AP34）

| Example： |  |
| --- | --- |
| IWBP34,353456789012345,080835,2,0900@1145,1300@1600# |  |
| Notice: |  |
| IW：Identifier BP34 : Command word 353456789012345: IMEI number and the unique ID of the device 080835 : command serial number 2: shows numbers of intervals, GPS working intervals are cancelled if the number is 0. 0900@1145，shows a set of GPS working time interval, 0900@1145 GPS is working from 09:00am to 11:45am and using 24 hour system. |  |
| device responds： | IWAP34,080835,2,0900@1145,1300@1600# 080835: command serial number 2,0900@1145,1300@1600 setting information to sync server |
| Notice: | This message applies to all end users. |

- Temperature upload interval（downlink：BPTE，respond：APTE）

| Example： |  |
| --- | --- |
| IWBPTE,353456789012345,123456,300# |  |
| Notice: |  |
| IW：Identifier BPTE : Command word 353456789012345: IMEI number and the unique ID of the device 123456: command serial number 300: 300 seconds |  |
| device responds： | IWAPTE,123456# 123456: command serial number |
| Notice: | This message applies to all end users. |